PACKET TRACER LAB GUIDE

Configure Named Standard IPv4 ACLs

| TARGET DEVICE Router R1 (Cisco IOS) | ACCESS CONTROL TYPE Named Standard IPv4 Access Control List (ACL) |
| --- | --- |
| SECURITY POLICY Restrict File Server access to PC1 and Web Server only | INSPECTION COMMANDS show access-lists ¦ show ip interface ¦ show run |

📌 LAB SCENARIO & OBJECTIVES: The senior network administrator has tasked you with configuring a standard named Access Control List (ACL) on router R1 to restrict access to a critical File Server containing backend web databases. Only the Web Manager workstation (PC1) and the Web Server are permitted to communicate with the File Server. All other host traffic targeting the File Server must be denied.

Addressing & Interface Reference Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway | Connected Role / Function |
| --- | --- | --- | --- | --- | --- |
| R1 | F0/0 | 192.168.10.1 | 255.255.255.0 | N/A | LAN Gateway (PC2) |
| R1 | F0/1 | 192.168.20.1 | 255.255.255.0 | N/A | LAN Gateway (PC0, PC1) |
| R1 | E0/0/0 | 192.168.100.1 | 255.255.255.0 | N/A | DMZ Gateway (Web Server) |
| R1 | E0/1/0 | 192.168.200.1 | 255.255.255.0 | N/A | Server LAN Gateway (File Server) |
| File Server | NIC | 192.168.200.100 | 255.255.255.0 | 192.168.200.1 | Protected Database Server |
| Web Server | NIC | 192.168.100.100 | 255.255.255.0 | 192.168.100.1 | Permitted Application Server |
| PC0 | NIC | 192.168.20.3 | 255.255.255.0 | 192.168.20.1 | General User (Denied File Server) |
| PC1 | NIC | 192.168.20.4 | 255.255.255.0 | 192.168.20.1 | Web Manager (Permitted Access) |
| PC2 | NIC | 192.168.10.3 | 255.255.255.0 | 192.168.10.1 | General User (Denied File Server) |

Part 1: Configure and Apply a Named Standard IPv4 ACL

Step 1: Verify Baseline Connectivity (Before Applying ACL)

• Open the Command Prompt on PC0, PC1, and PC2.

• Verify that all three workstations can successfully ping both the Web Server (192.168.100.100) and File Server (192.168.200.100).

• Confirm full reachability across all subnets before implementing access control filters.

PC0> ping 192.168.100.100 # Ping Web Server - Success (4/4) PC0> ping 192.168.200.100 # Ping File Server - Success (4/4) PC1> ping 192.168.100.100 # Ping Web Server - Success (4/4) PC1> ping 192.168.200.100 # Ping File Server - Success (4/4) PC2> ping 192.168.100.100 # Ping Web Server - Success (4/4) PC2> ping 192.168.200.100 # Ping File Server - Success (4/4)

Step 2: Create the Named Standard ACL on Router R1

• Access Router R1 and enter Global Configuration mode (configure terminal).

• Create a named standard access list named File_Server_Restrictions (exact spelling and case-sensitive).

• Add permit statements for Web Manager PC1 (192.168.20.4) and Web Server (192.168.100.100).

• Explicitly deny all remaining traffic using deny any.

R1> enable R1# configure terminal R1(config)# ip access-list standard File_Server_Restrictions R1(config-std-nacl)# permit host 192.168.20.4 R1(config-std-nacl)# permit host 192.168.100.100 R1(config-std-nacl)# deny any R1(config-std-nacl)# exit

⚠️ IMPORTANT SYNTAX & DESIGN RULES: ACL names in Cisco IOS are case-sensitive. The statements must be entered in the exact order shown. In Cisco IOS, standard ACLs filter traffic based solely on source IP address and should theoretically be placed as close to the destination as possible to avoid prematurely blocking legitimate traffic to other destinations.

Step 3: Verify ACL Contents Prior to Interface Application

• Return to privileged EXEC mode and run show access-lists.

• Verify rule sequence numbers (10, 20, 30) and confirm no IP typos exist.

R1# show access-lists Standard IP access list File_Server_Restrictions 10 permit 192.168.20.4 20 permit 192.168.100.100 30 deny any

Step 4: Apply the ACL to the Target Interface and Save Configuration

• Navigate into interface configuration mode for FastEthernet 0/1 (as specified in lab activity instructions).

• Apply the access list outbound using ip access-group File_Server_Restrictions out.

• Save the active running configuration to NVRAM.

R1(config)# interface FastEthernet 0/1 R1(config-if)# ip access-group File_Server_Restrictions out R1(config-if)# end R1# copy running-config startup-config Destination filename [startup-config]? [Enter] Building configuration... [OK]

📌 PRODUCTION BEST PRACTICE NOTE: In an actual operational production network, applying an access list to an active interface without prior maintenance scheduling is risky. In Packet Tracer labs, ensure the interface matches the curriculum scoring engine exactly.

Part 2: Verify the ACL Implementation & Policy Enforcement

Step 1: Verify Interface Binding and Configuration

• Execute show ip interface FastEthernet 0/1 to confirm the outgoing access list is active.

• Run show running-config to inspect the full interface and ACL definitions.

R1# show ip interface FastEthernet 0/1 FastEthernet0/1 is up, line protocol is up Internet address is 192.168.20.1/24 Outgoing access list is File_Server_Restrictions # <-- Verified active outbound Inbound access list is not set

Step 2: Execute Functional Traffic & Reachability Verification

• Test 1 (Web Server Reachability): PC0, PC1, and PC2 must all successfully ping Web Server (192.168.100.100).

• Test 2 (Authorized File Server Access): PC1 (Web Manager) and Web Server must successfully ping File Server (192.168.200.100).

• Test 3 (Denied File Server Access): PC0 and PC2 must be blocked when attempting to ping File Server (192.168.200.100).

# 1. Test from Permitted Workstation (PC1): PC1> ping 192.168.200.100 # Success (Reply from 192.168.200.100) # 2. Test from Denied Workstations (PC0 & PC2): PC0> ping 192.168.200.100 # Denied (Request timed out or Destination Host Unreachable) PC2> ping 192.168.200.100 # Denied (Request timed out or Destination Host Unreachable) # 3. Test General Web Server Access (All PCs): PC0> ping 192.168.100.100 # Success (Reply from 192.168.100.100) PC1> ping 192.168.100.100 # Success (Reply from 192.168.100.100) PC2> ping 192.168.100.100 # Success (Reply from 192.168.100.100)

Step 3: Inspect Packet Match Counters in ACL Statistics

• Execute show access-lists on R1 to verify hit counters corresponding to permitted and denied packets.

R1# show access-lists Standard IP access list File_Server_Restrictions 10 permit 192.168.20.4 (4 matches) 20 permit 192.168.100.100 30 deny any (8 matches)

Step 4: Check Results in Packet Tracer

• Click the Check Results button at the bottom of the Packet Tracer lab window.

• Ensure your completion percentage displays 100%.

• Review the Assessment Items tab to confirm all ACL statements, interface bindings, and directionalities are satisfied.

Standard vs. Extended IPv4 ACL Quick Comparison

| ACL Parameter | Standard IPv4 ACLs | Extended IPv4 ACLs |
| --- | --- | --- |
| Number Range | 1 - 99 and 1300 - 1999 | 100 - 199 and 2000 - 2699 |
| Named Syntax | ip access-list standard <name> | ip access-list extended <name> |
| Filtering Criteria | Source IP address only | Source & Destination IP, Protocol, Port numbers |
| Recommended Placement | As close to the DESTINATION as possible | As close to the SOURCE as possible |

📌 LAB COMPLETION SUMMARY: Lab completed successfully. You have created a named standard access list, enforced source-based filtering, bound the policy to the router interface, and verified policy compliance using ICMP ping tests and packet match counters.